Class Based TB Implementation Report

### Overview

A class-based verification environment was implemented for the MIPS Lite CPU to enable scalable, reusable, and coverage-driven verification. This environment replaces basic directed testing with a modular architecture that separates stimulus generation, monitoring, checking, and coverage.

### Verification Architecture

The testbench consists of the following components:

#### Generator

- Produces instruction transactions.

- Predicts expected ALU outputs based on instruction decoding.

- Sends transactions to driver and scoreboard via mailboxes.

#### Driver

- Provides timing synchronization.

- Acts as execution pacing element rather than direct signal driver.

- Helps emulate realistic CPU execution flow.

#### Input Monitor

- Observes instruction fetch interface.

- Displays PC progression and instruction execution.

#### Output Monitor

- Captures DUT outputs:

  - ALU result

  - Instruction execution state

- Sends observed data to scoreboard.

#### Scoreboard

- Compares expected ALU results with observed results.

- Maintains pass/fail counters.

- Generates final verification summary.

#### Coverage Collector

- Functional coverage implemented using SystemVerilog covergroups.

- Tracks opcode usage, register utilization, and ALU result distributions.

### Verification Achievements

- Successfully executed directed instruction sequences.

- Functional coverage collected for major datapath elements.

- Modular verification environment established for future expansion.

### Observed Issue (Scoreboard Failures)

The scoreboard currently reports mismatches due to:

- Pipeline latency misalignment between expected and observed outputs.

- Generator assumes zero-latency execution.

- Output monitor captures results after pipeline delay.

This causes all transactions to appear as failures even though waveform inspection confirms correct functionality.

### Planned Fix

Future improvements include:

- Pipeline stage latency modeling in scoreboard.

- Transaction tagging with cycle alignment.

- Enhanced synchronization between generator predictions and DUT outputs.

Conclusion

The class-based verification environment is functionally operational, provides meaningful coverage data, and establishes a scalable verification infrastructure. Remaining issues are limited to scoreboard synchronization rather than functional CPU errors.